The Model

The optimization model developed for the Town Hall is designed as a multi-criteria optimization problem. The goal of the solver is to determine the optimal system configuration and identify the ideal ratio between photovoltaic and solar thermal panels on the roof. In doing so, the algorithm optimizes a mathematical objective function composed of two competing primary objectives over a 25-year time horizon: maximizing the net present value (NPV) and minimizing the building’s total greenhouse gas emissions.

To enable the mathematical control of these objectives, the model includes a variable weighting function. The factor w can be used to flexibly determine whether the solver calculates a purely cost-optimized solution, a purely emissions-minimized solution, or a balanced compromise solution. The combined objective function Z, which is minimized by the solver, is:

minZ=w1-NPVKBaseline,Total+1-wEScenario,TotalEBaseline,Total

Z: The total value of the objective function that the solver minimizes.

w: The weighting factor, which can be chosen freely (0≤w≤1).

NPV: KBaseline,Total-KScenario,Total  The financial value added by the project over N years compared to the baseline, calculated as the difference between the baseline costs and the scenario costs

KBaseline,Total: The total energy costs of the Town Hall under current conditions (baseline) over N years, which serve as a financial benchmark.

EScenario,Total: Total emissions for the scenario over N years, including residual energy consumption and production-related plant emissions.

EBaseline,Total: The total CO₂ equivalent emissions of the Town Hall under current conditions (baseline) over N years.

Decision Variables and Constraints

The key decision variables in the model are the installed areas of the two solar technologies:

x= Area of the photovoltaic system in m²

y= Area of the solar thermal system in m²

The solver thus determines how much of the available roof area should be used for photovoltaics and how much for solar thermal systems. The total installable area is limited by the available or specified roof area:

x+y≤ALimit

In addition, no negative areas may be selected:

x≥0, y≥0

In the case of solar thermal systems, this also prevents the system from being oversized relative to the theoretical heat demand:

y⋅qST≤Qcurrent

Here, ALimit represents the maximum roof area available for installation, qST represents the specific annual solar thermal yield per square meter, and Qcurrent represents the current annual heat demand of the town hall.

Neben den Dachflächenrestriktionen werden im Modell alle Energieflüsse auf sinnvolle Werte begrenzt. Der PV-Eigenverbrauchsanteil scx,t sowie der Solarthermie-Nutzungsfaktor uSTy,tliegen jeweils zwischen 0 und 1. Außerdem werden verbleibender Strom- und Wärmebezug mit max0,⋅berechnet, sodass keine negativen Restbedarfe entstehen können. Wird die Budgetrestriktion aktiviert, gilt zusätzlich:

In addition to the roof area restrictions, all energy flows in the model are limited to reasonable values. The PV self-consumption share scx,t and the solar thermal utilization factor uSTy,t are each between 0 and 1. Furthermore, remaining electricity and heat consumption are calculated as \max0,⋅ so that no negative residual demands can arise. If the budget constraint is activated, the following also applies:

InvPV⋅x+InvST⋅y+KBatt≤Budget

InvPV= Investment cost in CHF per m² of photovoltaic system

InvST= Investment cost in CHF per m² of solar thermal system

KBatt= Battery cost in CHF

In this case, the sum of the PV investment, solar thermal investment, and optional battery costs must not exceed the specified budget. Battery costs are only taken into account if the battery option is enabled.

Baseline Functions

The baseline functions describe the reference state of the town hall without the use of additional photovoltaic or solar thermal systems. They serve as a basis for comparison in the economic and environmental assessment of the optimized scenarios. The analysis takes into account the building’s current electricity and heat demand, the respective energy prices, and the associated emission factors.

The annual baseline costs in year t are calculated based on electricity consumption, heat consumption, and annual fixed electricity costs:

KBaseline,t=Scurrent⋅Pelectricity,t+Qcurrent⋅Pheat,t+Felectricity,t

KBaseline,t= Baseline Cost in CHF in year t

Scurrent=  Current Electricity Consumption in kWh

Pelectricity,t= Electricity Price in CHF per kWh in year t

Qcurrent= Current Heat Consumption in kWh

Pheat,t = Heat Price in CHF per kWh in year t

Felectricity,t= Fixed Electricity Charges in CHF in year t

Over the entire project duration, the present value of the baseline costs amounts to:

KBaseline,Total=t=1NKBaseline,t1+it

i= Discountfactor

The annual baseline emissions are as follows:

EBaseline,t=Scurrent⋅EFelectricity+Qcurrent⋅EFheat

EFelectricity= Electricity emission factor (kg CO₂/kWh)

EFheat= Heat emission factor (kg CO₂/kWh)

For cumulative emissions over the entire period under consideration:

EBaseline,Total=t=1NEBaseline,t

Scenario Functions

The scenario functions, on the other hand, describe the situation after the new systems have been installed. They calculate the remaining electricity and heat demand after self-generation via PV and solar thermal systems. In addition, feed-in revenues, maintenance costs, system emissions, and, where applicable, battery effects are taken into account. This allows the model to determine the extent to which costs and emissions change compared to the baseline.

The annual costs of the scenario are calculated as follows:

KScenario,t=Felectricity,t+Sres,t⋅Pelectricity,t+Hres,t⋅Pheat,t+MPV,t⋅x+MST,t⋅y-PVexport,t⋅Pfeed-in,t

PVt=x⋅qPV⋅fPVt      PVt: the amount of electricity generated by the photovoltaic system in year t in kWh.

qPV= specific annual PV yield, kWh/(m²·y)

fPVt=max0,1-t-1dPV)     fPVt= Annual yield factor of the PV system, taking degradation into account dPV.

PVself,t=minPVt⋅scx,t,Scurrent)   PVself,t= PV electricity used on-site for self-consumption in year t in kWh.

scx,t= is the effective self-consumption share of PV production in year t. It depends on the PV area, the available roof area, and, optionally, the battery. The exact calculation is explained in the section on utilization rates.

PVexport,t=max0,PVt-PVself,t    PVexport,t= PV electricity exported to the grid in year t in kWh

Sres,t=max0,Scurrent-PVself,t    Sres,t= Residual grid electricity demand in year t in kWh

STt=y⋅qST⋅fSTt     STt= olar thermal heat produced in year t in kWh qST= Specific annual solar thermal heat yield per square meter kWh/(m2*y)

fSTt=max0,1-t-1dST)   fSTt= s the annual yield factor of the solar thermal system, taking degradation dST into account.

STuse,t=STt⋅uSTy,t    STuse,t= Usable solar thermal heat in year t, i.e. the share of produced solar heat that can be used by the town hall.

Hres,t=max0, Hcurrent-STuse,t   Hres,t= Residual heating demand in year t in kWh

PVexport,t=max0, PVt-PVself,t    PVexport,t= Self-generated PV electricity exported to the grid in year t in kWh

Pfeed-in,t= Feed-in tariff received for PV electricity exported to the grid in year t in CHF per kWh

MPV,t= Maintenance cost in CHF per m2 Photovoltaic per year

MST,t= Maintenance cost in CHF per m2 Solarthermie per year

KScenario,Total=I0+t=1NFelectricity,t+Sres,t⋅Pelectricity,t+Hres,t⋅Pheat,t+MPV,t⋅x+MST,t⋅y-PVexport,t⋅Pfeed-in,t1+it
I0=InvPV⋅x+InvST⋅y+KBatt

The annual emissions of the scenario:

EScenario,t=Sres,t⋅EFelectricity+Hres,t⋅EFheat+PVt⋅EFPV+STt⋅EFST

EFPV= Photovoltaic Emission factor (kg CO₂/kWh)

EFST= Solar thermal Emission factor (kg CO₂/kWh)

The following applies to cumulative emissions over the entire period under consideration:

EScenario,Total=t=1NEScenario,t

Inflation Assumption

An annual inflation factor can be enabled in the model. If this option is enabled, annual costs such as electricity prices, heating prices, fixed electricity costs, and maintenance costs increase each year by the specified percentage. The feed-in tariff, however, remains constant, as it is assumed to be contractually fixed with the grid operator. The initial investments are also not adjusted for inflation, as they are incurred at the start of the project.

Utilization Rates and Self-Consumption

For photovoltaic systems, it is taken into account that not all of the electricity generated can be used directly in the town hall. The self-consumption rate scx,tdescribes what proportion of the PV output in year t is used directly on-site. As the PV area increases, this proportion tends to decrease, as production more often exceeds the current electricity demand. If a battery is activated, the self-consumption share can increase, as part of the excess PV electricity can be temporarily stored and used later. In the model, the value is limited to the range between 0 and 1.

For solar thermal energy, it is similarly taken into account that not all of the heat generated can be fully utilized. The utilization factor uSTy,t describes the proportion of solar thermal production that can actually be used to meet heat demand. For small solar thermal areas, a high utilization rate is assumed, whereas for larger areas, the usable portion may decrease because at times more heat is produced than can be directly used or stored. This factor is also limited to a range between 0 and 1.

Taking these dynamic utilization rates and self-consumption shares into account is essential for the validity of the optimization model. Without these constraints, the solver would assume an unrealistic, linear scalability of yields. As a result, the model would theoretically completely over-engineer the town hall roof with solar components, since mathematically every additional square meter of area would also generate a proportional benefit in the objective function.

The degressively decreasing factors specifically model the real-world phenomenon of temporary surplus production. As a result, the model must balance the economic and ecological marginal costs of an additional unit of area against the decreasing marginal benefit. This supports a practical, demand-oriented sizing of the systems and reduces the risk of over-sizing in the simulated energy system.

Formal Representation of Utilization Rates

For photovoltaics, the self-consumption rate scx,t is modeled as an area-dependent function. Here, scSlider describes the self-consumption rate, which is assumed as the reference value for a PV system installed on the entire available roof area ARoof. When the battery option is activated, the dummy variable ActiveBatt jumps from 0 to 1.

scx,t=max0,min1,1-1-scSlider⋅xARoof+ActiveBatt⋅min0.5,γ⋅qPV⋅Bestimate⋅fBatttx

The remaining usable battery capacity in year t is described by the degradation factor:

fBattt=max0,1-t-1dBatt)

Here, dBatt is the battery's annual degradation factor. It describes the percentage by which the usable battery capacity decreases each year.

The estimated battery capacity is calculated based on the battery costs you entered:

BestimateKBatt=KBatt847,KBatt<1694020+KBatt-16940163.25,16940≤KBatt<30000100+KBatt-30000266.67,KBatt≥30000

The variable Bestimate describes the storage capacity derived from battery costs. A larger storage capacity allows for the storage of additional PV surplus, thereby increasing the proportion of self-consumption from the photovoltaic system.

γ is a calibration factor for the battery effect. It determines the extent to which an existing battery increases PV self-consumption in the model. A low value means that the battery has only a small additional effect on self-consumption. A high value means that the battery contributes more significantly to storing excess PV electricity for later use. γ can take values between 0 and 1.

For solar thermal systems, the utilization factor uSTy,t is modeled as an area-dependent function. Here, umin represents the minimum utilization rate assumed when the available roof area ARoof is fully utilized by the solar thermal system. The variable y denotes the installed solar thermal area, while fSTt accounts for the annual degradation of the solar thermal system.

First, the effective solar thermal area for year t is calculated:

yefft=y⋅fSTt

Based on this, the solar thermal utilization factor is calculated as follows:

uSTy,t=maxumin,min1,1-1-umin⋅yefft-1ARoof-1

For a very small solar thermal area of y=1 m2, a full utilization rate of 100% is assumed. As the effective solar thermal area increases, the utilization factor decreases gradually until it reaches the specified minimum value u_{min}. This accounts for the fact that larger solar thermal systems more frequently generate excess heat that cannot be fully utilized in the town hall.